# LOCAL AUTHORITY NOTICES • PLAASLIKE 0WERHEIDS KENNISGEWINGS

# LOCAL AUTHORITY NOTICE 67 OF 2017

NKANGALA DISTRICT MUNICIPALITY

BY-LAWS ON OUTDOOR ADVERTISING

These By-laws will apply for the Victor Khanye Local Municipality, Emakhazeni Local Municipality, Dr JS Moroka Local Municipality and Thembisile Hani Local Municipality. The Municipal Managers of these four Local Municipalities hereby publishes these By-laws on Outdoor Advertising in terms of Section 13(a) of the Local Government: Municipal Systems Act, 2000 (Act No. 32 of 2000), as approved by its Council, as set out hereunder.

TABLE OF CONTENTS

PART A:	DEFINITIONS, PROCEDURAL CLAUSES, APPLICATIONS, PROHIBITIONS AND LAW ENFORCEMENT

- Definitions

- Purpose of these By-laws

- Administration of applications

- Tariffs

- Enforcement and removal of signs and structures

- Withdrawal or amendment of Municipal approval, appeal procedure and complaints

- Entry and inspection

- Removal and disposal of signs

- Serving of notice

- Offences

- Responsible person

- Transitional provisions

- Short title

PART B:	GENERAL CONDITIONS AND AREAS OF CONTROL

- General requirements

- Design, construction and position on the site

- Maintenance

- Electrical and illumination

- Content, amenity and decency

- Road safety and traffic considerations

- Areas of control

- Position and spacing

- Exempted signs

- Prohibited signs and areas

# PARTC:	CRITERIA	FOR	DIFFERENT	TYPES	OF	ADVERTISING	SIGNS	AND ADVERTISEMENTS

- Billboards

- Gantry signs

- Roof signs

- Flat signs

- Advertising signs at educational facilities and sport stadiums and fields

- Service facility signs

- On-premises business signs

- Projecting signs

- Advertising signs on boundary walls and fences

- Painted advertisements

- Security advertising signs

- Balcony, veranda, canopy and under-awning signs

- Residential home-undertaking, farm names and community institution signs

- Vehicular advertising

- Construction site boundary signs

- Project and development signs

- Advertisement for the sale of goods and livestock

- Estate agent signs

- Portable or forecourt signs

- Banners and flags

- Posters

- Advertisements on litter bins

- Advertisements on public transport shelters

- Stack signs

- Street name signs

- Suburb name signs

# PART A:	DEFINITIONS, PROCEDURAL CLAUSES, APPLICATIONS, PROHIBITIONS AND LAW ENFORCEMENT

- Definitions

In these By-Laws, unless the context indicated otherwise -

"advertisement" means any visible representation of a word, logo, name, letter, figure, object, mark, symbol, abbreviation, light or any combination thereof with the object of transferring information which is visible from any street or public place, but does not include a road traffic sign.

"advertisement for sale of goods or livestock" means an advertisement announcing such a sale on land or premises not normally used for commercial purposes and may include an advertisement announcing auctions of household goods on residential properties, livestock or game on farms. It also includes the display of an advertisement announcing the proposed sale of property or land by means of a public auction to be advertised within the road reserve.

"advertisement on street furniture" means a poster which does not exceed 2,2 square metres in area, which is attached to street furniture and which has been approved by the Municipality;

"advertiser" means the person or organization whose product or service is being advertised, or whose name or image is mentioned or promoted in an advertisement.

"advertising" means the act or process of notifying, warning, informing, making known or any other act of transferring information in a visible manner.

"advertising sign or sign" means any advertising structure built or erected to display an advertisement, together with an advertisement displayed on the structure.

"advertising structure" means any screen, fence, wall or other physical structure erected to display an advertisement or which is in itself an advertisement or used to display an advertisement.

"affix" means to firmly secure, which may include painting where permitted, or any other means of affixing.

"animation" means a process whereby an advertisement's visibility or message is enhanced by means of moving units or pictures, flashing lights or similar devices; or containing a variable message.

"approved" means to obtain approval through a formal application process by the Local Municipality and "approval" has a corresponding meaning.

"area of advertisement" means the total area of that which constitutes the advertisement.

"area of control" refers to the degree of advertising control to be applied in a specific area, i.e. maximum, partial or minimum control as determined by the Local Municipality from time to time.

"area of jurisdiction" means the area under the control of the specific Local Municipality according to the legally determined and declared boundaries of the Municipality.

"area of maximum control" means an area which is deemed sensitive to visual disturbance and includes, but is not limited to, conservation areas and natural features inside the urban edge, passive recreation areas such as parks and non-commercial squares, scenic features and areas, historical and architectural sites, all residential areas, plots and urban small-holdings (which are proclaimed), home-undertakings within residential areas, rural-residential areas, specifically proclaimed heritage areas and buildings, and cemeteries.

"area of minimum control" includes, but is not limited to commercial areas, central business areas of towns, office blocks, shopping centres, industrial areas and transport nodes (such as taxi and bus ranks, railway stations, etc.).

"area of partial control" means an area that is characterized by a greater degree of integration and complexity of land use, including but not limited to commercial enclaves, shopping centres and office blocks in residential areas, commercial ribbon development, school and educational facilities (eg. Universities, Technicons, Colleges), sport stadiums and fields, and rural-commercial areas.

"balcony, veranda, canopy and under-awning sign" means a sign not extending above, below or beyond any extremity or a parapet wall, balustrade, railing, beam or fascia of a roof structure without walls, pillar, column or post supporting a roof structure without walls, and affixed flat to or painted on it, or printed on the fabric of a blind.

"banner" means a piece of cloth or similar material upon which an advertisement is displayed in such a manner as to be fully legible in windless conditions, attached to one or more ropes, poles or attached to buildings or to specific structures, but excludes banners carried as part of a procession.

"billboard" means any screen or board larger than 4,5m2 and not exceeding 81m2, supported by a free-standing structure, which is to be used or intended to be used for the purpose of posting, displaying or exhibiting an advertisement and can be classified as a small, large or super billboard. This can be attached to a structure manufactured specifically for advertising, or to a

structure of any other form, used to attach the advertisement to, which can include product replicas, towers, pylons and bridges. The main function of a billboard is to advertise non-locality bound products, activities or services.

"bit of information" refers to the basic unit for measuring the length of advertising messages and may consist of letters, digits, symbols, logos, abbreviations or graphics of any nature.

"by-law" means the Local Municipality By-laws on Outdoor Advertising, addressing the control of outdoor advertising in the area.

"candela" is the standard SI unit of luminance intensity relating to the illuminating power of a light source in a given direction.

"centre of economic activity (CEA)" means an urban area of high economic activity and includes all business districts, regional and neighbourhood shopping centres and it can also include a business or group of businesses outside of urban areas which may include farm stalls, roadside service areas, accommodation facilities, food services, industries and cottage industries as well as shops and other commercial facilities.

"centre point of intersection" means the point of contact between the centre lines of two roads.

"clear height" means the minimum vertical distance from the ground, road or surface level, as the case may be, to the bottom of the advertisement and / or to the advertising sign, whichever is the smallest.

"combination sign" means an advertising sign comprising a number of smaller, individual advertisements, usually displaying different products or services, placed next to each other on a single free-standing advertising structure specially designed to accommodate more than one advertisement presented on an on-premises business advertising sign.

"community awareness signs" means signs that make the community aware of a specific aspect that need attention or educate the community on something important, and will be in the size and format of a small billboard.

"construction site boundary sign" means an advertising sign of vinyl mesh or similar material attached to the boundary wall or fence or to a structure, to cover an unsightly construction site, an unoccupied building under construction, renovation or maintenance, or to advertise some details of the development.

"Council" means the relevant Local Municipality and includes the Mayoral Committee or any officer employed by Council, acting by virtue of any power vested in the Council in connection with these By-laws and delegated to him / her.

"Council land" means any portion of land including road, road reserve, street, thoroughfare, bridge, subway, footpath, sidewalk, land, square, open space, garden, park or enclosed place, erf, site, etc. vested in and/or owned by the Municipality.

"cut-outs" means letters, packages, figures or mechanical devices attached to the face of an outdoor advertising sign, which might extend beyond the rectangular area for greater attention value, can provide a three-dimensional effect and are also commonly known as add-ons or embellishments.

"development sign" means an advertising sign displaying and advertisement which describes the type of development being carried out on a construction site, and can include a pictorial representation, and contain the contact details of the developer or his agent.

"display period" means the exposure time during which the individual advertising message is on display.

"educational facility" means a primary school, secondary school or tertiary institution.

"election" means either Central Government, Provincial Government or Municipal elections and by-elections inclusive of the registration process and referendum held from time to time.

"electronic billboard" means an advertising sign not exceeding 18m2 in advertisement area, which has an electronically controlled, illuminated display surface, which allows all or a portion of the advertisement to be changed, animated or illuminated in different ways.

"entertainment area" means an area, the main purpose of which is to be used as a park, sports field, barbeque area or for other recreational purpose.

"erf" means an erf, stand, lot, plot, agricultural holding or similar land entity registered in a deeds registry.

"estate agent sign" means an advertising sign that is temporary displayed to advertise the fact that land, premises, development or other forms of fixed property are for sale, to let, sold or on show.

"existing sign" means an advertising sign, which has been erected.

"face change" means that the advertisement content has been changed.

"flag" means a material upon which an advertisement is displayed and which is attached to a single free-standing flag staff, pole or flagstaff projecting vertically from premises or projecting

vertically, horizontally or at an angle from a building or boundary wall or boundary fence of a specific business property.

"flat sign" means any advertising sign affixed to any external wall of a building used for commercial, office, industrial or entertainment purposes, but excluding a parapet wall, balustrade or railing of a veranda or balcony, which at no point projects more than 300mm from the surface of such a wall and which may consist of a panel or sheet or of individual numbers, letters or symbols.

"free-standing sign" means any immobile advertising sign, which is not attached to a building or to any structure or object not intended to be used for the primary purpose of advertising.

"gantry" means a ground-mounted structure that spans over the entire width of a roadway/ road surface (portal gantry) or a structure that spans over part of a roadway/ road surface (cantilever gantry).

"height of advertising sign" means the maximum vertical distance from where the foundation of the sign starts, to the top of the advertisement and / or the advertising sign, whichever is the highest. The natural ground level may not be altered for the purpose of erecting the sign.

"home-undertaking" means the practice of an activity at a dwelling-place with the aim of deriving an income there from.

"illegal sign" means any advertising sign, advertisement, or poster, painted, pasted, affixed, displayed, exhibited, posted or erected without approval from the Local Municipality or whose display, exhibition or erection is contrary to or in conflict with the provisions of these By-laws.

"illuminated advertisement or advertising sign" means an advertisement or advertising sign which has been installed with electrical or other power for the purpose of external or internal illumination, either continuously or intermittently of the advertisement displayed or advertising sign.

"large billboard" means any static billboard larger than 18m2 up to and including 40m2 in area of advertisement.

"locality-bound advertisement" means an advertisement displayed on a specific site, premises or building and which refers to an activity, product, service or attraction located, rendered, sold or provided on that premises or site or inside that building

"luminance" is a measure of how bright an illuminated area appears to the human eye and is measured in candela/m2.

"m" means metre.

"mm" means millimetre.

"Municipality" means the relevant Local Municipality (LM) or Nkangala District Municipality (NDM), whichever is relevant.

"municipal area" means the jurisdiction area of the Municipality.

"municipal owned land" has a corresponding meaning to "Council land".

"National Road Traffic Act" means the National Road Traffic Act, 1996 (Act No. 93 of 1996), or as amended from time to time.

"natural area" means an area outside an urban area which is in a natural state or is of high scenic value, and includes, but is not limited to, national parks, game reserves, nature reserves, scenic corridors and landscapes.

"non-locality bound advertisement" means any advertisement displaying any information over and above the name, logo and nature of the business on that premises, and which may include any advertisement content and can include advertising of products for sale of any kind.

"on-premises business sign" means a sign on a specific site or premises aimed at identifying and locating businesses and industries, and may only contain the name, logo and nature of the business on the premises. It excludes residential or community advertising signs.

"outdoor advertising" means the act or process of notifying, warning, informing, making known or any other act of transferring information in a visible manner and which takes place out of doors.

"overhang" means the physical part of an advertising sign / structure hanging or projecting over a boundary, shoulder or roadway.

"owner of the advertising sign" means the person who owns the advertising sign which is erected, or will own the advertising sign once it has been erected, or any person who has a right to or share in the ownership of the advertising sign.

"owner of the advertising structure" means the person who owns the advertising structure, or will own the structure once it has been erected, or any person who has a right to, or share in the ownership of the advertising structure.

"owner of the land" means the person who owns the land on which the advertising sign or structure is, or will be erected, or any person who has a right to or share in the ownership of the land.

"permanent sign" means advertising signs or advertisement erected for a period of more than 30 days.

"perpendicular" means with a 90° angle to the existing building or road at the position of the advertisement or advertising sign.

"person" means both natural and juristic persons, and includes both genders and a reference to any one gender shall include a reference to the other gender.

"portable or forecourt sign" means a moveable free-standing temporary advertising sign displayed on a forecourt of a business premises or sidewalk in front of business premises to draw attention to commercial services, goods for sale or other services available at the premises.

"poster" means an advertisement displayed or affixed to a receptacle, on a hard board, or directly affixed against an electrical street light pole standard, inside the road reserve.

"prohibited sign" means an advertisement, advertising structure or advertising sign, which is not permitted, or does not conform to advertising By-laws and can not be approved.

"project sign" means an advertising sign displaying an advertisement with information regarding the relevant contractor(s) and / or consultant(s) involved in the construction project and displayed on the construction site.

"projecting sign" means any advertising sign, attached to and protruding from a building which is used for commercial, office, industrial or entertainment purposes and which projects more than 300mm from the surface of the main wall and is affixed at a right angle to the street line.

"property" means any piece of land whether registered or not in a deeds registry as an erf, lot, plot, farm, stand or agricultural holding.

"public body" means any authority or any statutory body acting on behalf of the government.

"public place" means any road, street, thoroughfare, bridge, subway, foot pavement, footpath, sidewalk, lane, square, open space, garden, park or enclosed space vested in and/or owned by The Municipality.

"public road" means a road which the public has the right to use.

"receptacle" means a fixed poster display holder to be fixed to street light poles.

"residential purposes" means the use of a building as a dwelling house, two or more dwelling units, a hostel, a boarding house and a residential club.

"road" means a public road which includes the shoulder, the land of which the road consist or over which the road extends, and anything on that land forming part of, connected with, or belonging to the road.

"road authority" means the authority having the control and jurisdiction over a specific road, whether it be a national, provincial, or municipal road.

"road island or traffic island" means an area demarcated on a roadway by means of painted lines, stones, kerbs or by other means with the intention of preventing vehicles from standing or being operated in that area.

"road median" means the area separating traffic lanes on a roadway.

"road reserve" means the full width of a public road, including roadways, shoulders, sidewalks, the air space above it and all other areas from boundary to boundary.

"road reserve boundary" means the proclaimed boundary forming the outer edge of the road reserve.

"road traffic sign" means any road traffic sign as defined in the National Road Traffic Act, 1996 (Act No. 93 of 1996).

"roadway" means the portion of a road, street or thoroughfare intended for vehicular traffic, which is between the edges of the travelled way and includes the surfaced shoulder.

"roof sign" means an advertising sign on the main roof of a building which is used or partly used for commercial, office, industrial or entertainment purposes.

"rural area / landscape" means and area forming a transition between developed urban areas and relatively unspoiled areas and includes, but are not limited to agriculture land, farm land, rural small holdings, un-proclaimed township areas, rural-residential areas, rural-commercial areas and areas outside the urban edge.

"rural-commercial area" means small commercial shops or home-undertakings located in a rural-residential area between rural residential erven, or next to each other to form a small commercial area in a rural township, which is located in a rural-residential area.

"rural-residential area" means a rural area (not officially proclaimed as urban area) with official or self-determined larger erven with residential houses, where the erven is smaller than small holdings or plots, with a rural atmosphere. Small shops or home-undertakings can also be part of this area.

"security sign" means an advertising sign with an advertisement for neighbourhood watch, farm watch, security schemes and other similar schemes, and includes an advertisement containing the name, address and telephone number of a security company contracted to protect the premises on which the advertisements is displayed.

"service area" means an area with direct access from a municipal or provincial road in which facilities and services such as petrol and diesel sales, restaurants, fast food outlets, toilets, playgrounds and picnic spots may be provided for motorists.

"service facility sign" means an advertising sign or advertisement at a filling station, roadside rest and service facility referring to the types of services provided at such facility.

"shoulder" is the outer portion of the roadway which, whether surfaced or not, does not normally constitute part of the travelled way.

"sidewalk" means that portion of a road reserve intended for the exclusive use of pedestrians.

"signs incorporated into the fabric of a building" means a sign incorporated in and forming an integral part of the fabric of a building, in balance with the scale of the building.

"site" means the erf, plot, or any other piece of land on which the advertising sign, advertising structure or advertisement is, or is to be erected or mounted.

"small billboard" means a static billboard larger than 4,5m2 up to and including 18m2 in area of advertisement.

"special consent" means to be considered with special attention, addressed all relevant aspects and investigated in detail by the relevant authority.

"stack sign" means a free-standing advertising sign in the road reserve displaying a combination of advertisements of several businesses on separate panels.

"street furniture advertisement" means advertisement on public facilities and structures which are not intended primarily for advertising and includes seating benches, planters, litter bins, public transport shelters, but excludes road traffic signs, traffic lights, street lights or any other road related structures.

"street name sign" means a pole-mounted, illuminated street name sign displayed in combination with a double sided, internally illuminated advertising sign, to be provided according to requirements as set out in the Southern African Development Communities Road Traffic Signs Manual, November 1997 (SADC RTSM).

"suburb name sign" means a pole-mounted road traffic sign (GL2 road traffic sign) at the entrance to a town or suburb that may carry an advertisement panel, to be provided according to requirements as set out in the Southern African Development Communities Road Traffic Signs Manual, November 1997 (SADC RTSM)..

"super billboard" means a static billboard larger than 40m2 up to and including 81m2 in area of advertisement.

"tariff" means the appropriate monetary charge, tariff or fee determined by the relevant Local Municipality.

"temporary sign" means an advertisement or advertising sign displayed for a maximum period of 30 days or less as determined by the Local Municipality.

"township" means an area divided into erven or plots, whether with or without public open spaces, and into streets bounded by the erven, plot or open spaces, and established or recognized as a township under any law.

"urban area" means proclaimed township area, or built-up areas within the Municipal area.

"vehicular advertising" means advertising on self-driven vehicles which are usually moving on land or water, including taxi's, buses, trains and delivery vehicles.

# Purpose of these By-laws

  - These By-laws are designed to regulate and control outdoor advertising in, on or visible from public roads within the area of jurisdiction of the Municipality, whether or not such sign is erected on private property. These By-laws apply to all outdoor advertising signs in this area and are binding on all persons, including the State, seeking to display or erect advertising signs or advertisements.

  - Unless these By-law provide specifically that the approval of the Municipality is required for a particular advertising sign, such approval is not required, but all of the conditions of these by­ law applicable to that type of advertising sign must be complied with.

  - Compliance with these By-laws does not override the necessity or obligation to comply with the requirements of any other law governing any matter covered by these By-laws.

# Administration of Applications

  - A person shall not display or erect any advertising sign, sign or advertisement, structure or device as a sign, without having obtained the written approval of the Local Municipality. The provisions of this Section shall not apply to signs deemed exempt or prohibited.

  - These By-laws are to be effectively and consistently administered, scheduled inspections of signage are to be held and offenders are to be fined or prosecuted. Nothing contained in these By-laws or any other law shall render the Municipality liable to any claim for damages for any failure to effectively and consistently administer these By-laws, to carry out scheduled inspections of signage or to fine or prosecute any offenders.

  - Applications for the erection or display of advertising signs or advertisements shall be accompanied by a completed application form, signed by all the relevant parties as required, with all the required documents attached, as stipulated on the application form, accompanied by the prescribed fee, as determined by the Local Municipality from time to time. Applications shall be submitted to the relevant office of the Local Municipality.

  - No sign for which approval has been granted may be erected otherwise than in accordance with any plan, drawing or other document approved by the Local Municipality or otherwise than in accordance with any condition imposed by the Local Municipality. The Municipality shall be notified in writing once any approved sign has been erected.

  - Every approval given by the Municipality shall be for a specified period only. Approval for display shall not be granted for an indefinite period. A new application shall be submitted for any additional period, provided that the Municipality shall be under no obligation to grant an approval for any additional period.

  - Where any advertising sign is in the road reserve of, or will be visible from a provincial or national road, the required comments and/or approval, whichever is applicable, for the erection or display of such advertising sign is required from the relevant road authority, after Municipal pre-approval has been obtained. Such a sign may not be erected or displayed without the final written approval of the Municipality and the required favourable comments and/or approval, whichever is applicable, of the relevant road authority.

  - The Local Municipality shall be entitled to appoint, within the framework of the law, private contractors or consultants to assist with any other work required to be done in terms of these By-laws, except law enforcement.

# Tariffs

  - Every person who applies to the Municipality for permission of an advertising sign or advertisement to be displayed, must on making the application, pay to the Municipality the tariff determined therefore, and no application will be considered until such tariff has been paid.

  - The set of rates as drawn up by Municipality and revised from time to time, as appropriate, shall apply.

# Enforcement and removal of signs and structures

  - If any sign is so displayed that, in the opinion of the Local Municipality, is detrimental to the environment or to the amenity of the surrounding areas or is otherwise in contravention of this By-law, the Local Municipality shall serve a notice on the responsible person to remove such sign or carry out such alteration thereto or do such other work as may be specified in such notice within the time specified in the notice.

  - If the responsible person fails to comply with an instruction contained in a notice, the Local Municipality may remove and destroy such sign. The Local Municipality may issue or send a spot fine with the notice. The Local Municipality will not be held responsible or be required to compensate any person in respect of removing advertising signs, where damage or loss was incurred due to its removal or destroying. Failure to respond to, or act upon, such notice shall be deemed to be an offence as provided in Section 10.

  - Notwithstanding the provisions of this Section, if the advertisement contravenes these By­ laws and is erected on, attached to, or displayed on any property of, or under the control of the Council, Council may, without serving any notice, remove any such advertising or structures from the premises, if in the opinion of the Local Municipality, such an advertisement poses and immediate danger to the general public.

  - Any costs incurred by the Local Municipality in removing and storing a sign, or doing alterations or other works in terms if this Section, will be recovered from the responsible person. Neither the Local Municipality nor the Municipal Manager will be liable for damages of whatever nature arising from the confiscation, removal, or disposal of the sign.

# Withdrawal or amendment of Municipal approval, appeal procedure and complaints

  - Any approval lawfully obtained from the Local Municipality to erect an advertisement or advertising sign, shall automatically lapse if the advertising structure is not erected within 6 months from the date of such approval. If the structure is erected, but an advertisement is not displayed within 3 months from date of erecting the structure, the Municipality will then serve a notice to the owner requiring him, at his own cost, to remove the structure.

  - The time periods referred to in subsection (1) may be extended upon good cause shown but Council is not obliged to grant such extension period as requested.

  - Any applicant who is not satisfied with any decision taken by the Local Municipality may appeal in writing against such decision by giving written notice of the appeal and reasons thereof to the Municipal Manager within 21 days of the date of the notification of the decision.

  - The Municipal Manager must, within a period of 90 days, submit the appeal to the appropriate appeal authority. The Municipal Manager or the relevant delegated appeal authority shall hear the appeal including any oral or written submissions from interested parties, and inform the applicant of its decision, which shall be final, and the reasons therefore.

# Entry and inspection

The Municipality will be entitled, through its duly authorised officers, to enter into and upon any premises, at any reasonable time for the purpose of carrying out any inspection necessary for the proper administration and enforcement of these By-laws.

# Removal and disposal of signs

  - The Municipality shall be entitled to, without notice to the owner thereof and without recourse to court, remove any advertisement or advertising sign or any part thereof, which is in a dangerous or unsafe condition or has been allowed to fall into a state of disrepair; constitutes a danger to any person; has been displayed or erected upon Municipal-owned property or property which vests in the Municipality in terms of any law, impedes the visibility of or restricts the proper functioning of any traffic sign or traffic light or can cause road safety problems, is prohibited in terms of this By-laws. Provided that no person shall have any claim against the Municipality or any employee, official or agent of the Municipality in respect of any actions performed in good faith under this subsection; provided further that the Municipality shall be entitled to recover all costs incurred by it in removing any advertisement or advertising sign pursuant to this subsection.

  - If any advertising sign is displayed so that, in the opinion of the Municipality, it is detrimental to the environment or to the amenities of the neighbourhood, or otherwise in contravention of any requirement of or prohibited by these By-laws, or has fallen into a state of disrepair, the Municipality may serve a notice on the owner of the advertising sign requiring him at own cost to remove such advertising sign or to carry out such alteration thereto or do such work as may be specified in such notice, as the case may be, within a time specified in the notice.

  - If a person fails to comply with a request contained in such notice the Municipality may remove such an advertisement, advertising sign or advertising structure at the expense of the owner.

  - The Municipality shall be entitled to recover any legal fees incurred by it in the exercise of its powers and or functions.

  - If any poster, advertising sign, advertising structure or advertisement has been removed and stored by the Municipality for more than 3 months, and has not been claimed by the owner, the Municipality may dispose of the sign, structure or advertisement. The Municipality may charge a storage fee for such signs that were removed and stored, which is payable before the sign can be reclaimed.

# Serving of Notice

Where any notice or other document is required by these By-laws to be served on any person, it must be deemed to have been properly served if served personally on him or on any member of his household apparently over the age of eighteen years or at his place of residence or on any person employed by him at his place of business, or if sent by registered post to such persons residential or business address as it appears in the records of the Municipality, or if such person is a company or closed corporation or a trust, if served on any person employed by that company, closed corporation or trust, at its registered office or sent by registered post to such an office.

# Offences

Any person who contravenes or fails to comply with: any provisions of these By-laws; requirement set out in a notice issued and served on him in terms of these By-laws; conditions imposed in terms of these By-laws; or knowingly makes a false statement in respect of any application in terms of these By-laws; will be guilty of an offence and will on conviction be liable to a fine not exceeding R20 000,00 or, in default of payment, to imprisonment for a period not exceeding twelve (12) months, and in the case of a continuing offence to a fine not exceeding R500,00 for every day during the continuance of such offence, and for a second or subsequent offence he will be liable on conviction to a fine not exceeding R10 000,00, or in default of payment, to imprisonment for a period not exceeding six (6).

# Responsible person

  - If any person is charged with an offence relating to advertising signs:

    - it shall be deemed that such person either displayed the advertising sign or caused or allowed it to be displayed;

    - the owner of any land or building on which any advertising sign was displayed, shall be deemed to have displayed such a sign, advertising structure or poster, or caused or allowed it to be displayed;

    - any person who was either individually or jointly, with any other person responsible for organizing, or is in control, of any meeting function or event to which a sign or poster relates, shall be deemed to have displayed every sign or poster displayed in connection with such meeting, function or event or to have caused or allowed it to be displayed; and

    - any person whose name appears on a sign, advertising structure or poster or to have caused or allowed it to be displayed, unless the contrary is proved.

# Transitional provisions

  - Where an advertisement has been displayed or an advertising sign or structure has without formal approval been erected prior to the date of commencement of these By-laws, which, in terms of these By-laws, may not be so erected or displayed without the approval of the Municipality, the owner of the advertisement / advertising sign must apply to the Municipality, as the case may be, for such approval, within 90 days of such date of commencement of these By-laws, failing which the advertisement / advertising sign must be removed forthwith.

  - If approval for an advertisement/ advertising sign contemplated in subsection (1) has been refused, the owner must remove it within 30 days of receipt of notification of such refusal. Where such a notification has been posted by registered post, the owner will be deemed to have received it eight days after posting thereof, subject to the provisions contained in subsection 6 (3).

  - Where an advertising sign has been legally, correctly erected or displayed before the date of commencement of these By-laws with a formal approval and/or contract from any former legal authority which by that time had the legal rights to give such approval and / or contract, which in terms of these By-laws may not be so erected, an agreement must be reached between the Municipality and the owner of the structure, regarding the correction, removal, relocation or replacement of the advertising sign to ensure compliance with these By-laws. Proof of this prior approval and/or contract has to be submitted to the Municipality on request. After agreement has been reached a new approval and/or contract, if applicable, shall be compiled between the Municipality and the owner of the structure.

# Short title

These By-laws are called the By-Laws on Outdoor Advertising, and come into operation on a date to be announced by notice in the Provincial Gazette.

# PART B:	GENERAL CONDITIONS AND AREAS OF CONTROL

- General requirements

  - No advertisement, advertising sign or advertising structure may:

    - in the opinion of the Local Municipality constitute a danger to any person or property;

    - be erected or approved for an indefinite period;

    - be erected without written approval where such approval is required by any Act or Regulations, By-law or any other law;

    - be detrimental to the environment or to the amenity of a human living environment by reason of size, shape, colour, smell, texture, intensity of illumination, quality of design or materials or for any other reason. Structures and signs in rural areas must not be of high impact, neon colours, or luminous colours; or

    - be permitted that emits a noise, sound, smoke, smell, odours or similar sensory stimuli.

  - The Municipality may increase the minimum spacing between advertising signs or advertisements, or place further restrictions on the position, size and content of any advertising sign or advertisement, if considered necessary, in the interests of road safety or environmental impact.

  - No advertisement may display the Municipality's corporate identity and/or Coat of Arms or any aspect of these registered logo's without prior consultation and written approval by the Municipality.

  - All policies, criteria or requirements addressing advertising signage within the Municipality need to be adhered to.

  - Signage which was not categorized and for which provision was not made in these By-laws, and which is not prohibited, will be addressed on an ad-hoc basis and considered by the Municipality.

# Design, construction and position on the site

  - Any advertising sign must, in the opinion of, or to the satisfaction of the Municipality:

    - be neatly and properly constructed, executed and finished in a workmanlike manner with durable materials, in accordance to the National Building Regulations and Building Standards Act, (Act No. 103 of 1977), or as amended from time to time;

    - not be detrimental to or have a negative aesthetic impact on the amenity of the neighbourhood, urban design, streetscapes or the character of the surrounding area, in any way;

    - be rigidly and securely attached, supported or anchored in a safe manner;

    - must be constructed and located at a height that discourage vandalism.

  - Any advertiser or contractor must:

    - have all exposed metalwork of any advertising structure painted or otherwise treated to prevent corrosion and all timber treated to prevent decay;

    - take measures to prevent the entry of water into and the accumulation of water or moisture on or in any advertising sign or any part of its supporting framework, brackets or other members.

  - No person may, in the course of erecting or removing any advertising sign, advertisement structure or device, cause damage to the environment, including any tree or vegetation or, electrical standard or bulk service or other public installation or property.

  - If an advertising sign contains glass, it must adhere to the following:

    - all glass used in advertising signs (other than glass tubing in neon and similar advertising signs) must be safety glass at least 3 mm thick; and

    - glass panels used in advertising signs must not exceed 0,9m2 in area, each panel being securely fixed in the body of the advertising sign, structure or device independently of all other panels.

  - Before any advertising structure is erected, it must be considered by the Municipality, whether it is suitably positioned and orientated.

  - No advertisement, advertising structure or advertising sign may;

    - obstruct any window or opening provided for the ventilation of a building or obstruct any stairway or doorway or other means of exit from a building or prevent the movement of persons from one part of a roof to another part; or obstruct any fire escape or the means of egress to a fire escape;

    - be painted on a boundary wall in an area of maximum control, or on bridges, water towers, reservoirs or silos;

    - exceed the prescribed height in terms of these By-laws or the permitted height on that specific site or on the surrounding or adjacent sites in terms of the relevant Town Planning Scheme or any other relevant document, as amended from time to time, or the approved zoning for the site of that specific site or the direct surrounding or adjacent sites, unless a relaxation has been obtained in terms of such Town Planning Scheme or any other relevant document, or unless specifically approved by the Municipality;

    - encroach on the building restriction area unless a relaxation has been obtained in terms of the relevant Town Planning Scheme or any other relevant document, as amended from time to time, and / or in terms of all the requirements from the other relevant road authorities;

    - be erected within or over any building line or servitude, unless specific approval has been granted by all the relevant authorities;

    - exceed the minimum clearance with regard to overhead power lines as prescribed in regulations 15 of the Electrical Machinery Regulations (No R1593 in GG11458 of 12 August 1988 ). (Permission must be obtained from the relevant supply authority before any advertising structure may be erected in or close to a power line servitude);

    - unreasonably obscure, partially or wholly, any advertising sign or advertisement owned by another person previously legally erected and legally displayed; and

    - in any way be altered, moved or re-erected, nor may any alteration be made to the electrical wiring system of such sign except for the purpose of renovating or maintenance, without the further approval of the Municipality.

  - All free-standing advertising signs located adjacent to a road reserve boundary of any road except a Municipal road, must maintain a minimum distance from the road reserve boundary equal to the height of the advertising sign, measured from the nearest pole to the road reserve boundary, except if a formal building line relaxation has been obtained from or another distance closer to the road reserve boundary was approved by the relevant road authority.

  - All advertising signs or advertisements located directly adjacent to a national or provincial road reserve, will be considered in terms of the criteria and requirements as set by that specific road authority.

  - If required by the Municipality, the structural design of an advertising structure must be certified by a professional structural engineer with acceptable relevant experience. The engineer must satisfy the Municipality that the structure will be adequate to secure, fix or support any advertisement, or screen to resist all loads and forces to which the advertising sign, structure, advertisement or screen may be exposed and the sufficiency of the margin of safety against failure, in compliance with the provisions of regulation B1 of the National Building Regulations published under Government Gazette No. 9613, dated 1 March 1985, and as amended from time to time.

- Maintenance

  - Any advertising sign as permitted must, on a regular basis, be maintained in good repair in a safe condition and according to the highest standards as regards quality of structures, posting and sign-writing.

  - The owner of any land or building on which an advertising sign, structure or advertisement is displayed or erected, or attached, and the owner of any such advertising sign, structure or advertisement shall be jointly and severally responsible for the maintenance thereof in a safe and proper condition, maintaining the surrounding area in a neat and tidy state and the

cleaning and repainting of any such advertising sign, structure or advertisement. The owners will be liable for the consequences of not doing so, and must undertake at least one annual inspection thereof in order to comply with the Municipality's requirements.

  - All advertising signs or any part thereof must be secured in a manner so as to not constitute a danger to the public. The land owner on which property such is located and the owner of the advertising structure, assume all responsibility and liability, indemnifying the Municipality against any claim which may arise. When the Municipality is the land owner, the responsibility and liability is with the owner of the advertising structure and the Municipality is to be indemnified by the owner of the advertising structure.

  - No sign may be erected or maintained in such a manner as to impede on landscaping, causing trees and other plants to be removed or trimmed to ensure that the advertising sign, structure or advertisement remain visible, without the consent of the Municipality. The cost thereof will be for the account of the applicant.

# Electrical and illumination

  - Every illuminated advertising sign and every advertising sign in which electricity is used:

    - must have power cables and conduit containing electrical conductors positioned and fixed so that they are not unsightly in the opinion of the Municipality;

    - must be constructed of material which is not combustible;

    - must be provided with an external switch in an accessible position at a height of at least three metres from the ground whereby the electricity supply to the advertising sign may be switched off;

    - must be wired and constructed in accordance with and subject to the provisions of the Municipality's requirements;

    - may not be connected to any electricity supply without the prior written permission of the relevant electricity supply authority. It must be in accordance with the provisions of the "Standard Regulations for the Wiring of Premises" or any other standard rules for the electrical wiring of premises or structures. Such proof of permission must be submitted if requested; and

    - which is likely to interfere with radio reception must be fitted with efficient suppressers.

  - The following maximum luminance levels per square metre for all classes of advertisements is permitted (as permitted by the International Commission on Illumination):

Illuminated area Less than 0,5 m2

Between 0,5 m2 up to 2 m2 Between 2 m2 up to 10 m2 10 m2 or more

Maximum luminance 1 000 candela/m2

800 candela/m2

600 candela/m2

400 candela/m2

  - The light source emanating from floodlights or light not meant for illumination, may not be visible to traffic travelling in any direction. Floodlighting must be positioned to ensure effective distribution and minimise light wastage or "spill".

  - Illumination on an advertisement or advertising sign is not permitted unless the area around the sign is illuminated. Illumination is further only permitted if it does not lead to unsafe driving conditions, does not have a detrimental effect on the visibility of road signs not illuminated, or on the surrounding area, does not constitute a road safety hazard or cause undue disturbance. No animation is permitted, except for electronic billboards.

  - An electronic advertisement or advertising sign may not inhibit the view of or cause discomfort or confusion to a driver or pedestrian or be in the direct line of sight of a traffic light.

  - An electronic advertisement must be static for at least 5 seconds per advertisement.

  - No advertisement or advertising sign may, if illuminated, be erected in such a way that it may have a detrimental effect on the amenity of a residential building on a residential zoned erf or, in the opinion of the Municipality, could be detrimental to the character or amenity of the neighbourhood.

# Content, amenity and decency

  - Advertisements positioned along roads and specifically targeting the road user must be concise and legible and must comply with the following requirements:

    - Bit values shall be calculated as follows per element of an advertisement:

| Words of up to eight letters, inclusive | 1,0 bit |
| --- | --- |
| Words of more than eight letters | 2,0 bits |
| Words such as "a, the, than, and, an" | 0,25 bits |
| Numbers of up to four digits, inclusive | 0,5 bits |
| Numbers of five to ten digits | 1,5 bits |
| Symbol, logo or graphic | 0,5 bit |
| Background graphics (depending on the destructiveness of the graphics) | 1,0 to 3,0 bits |

    - On any sign, the number of bits and size of the text should adhere to the following:

| Speed of the road | Bits allowable | Minimum size and |
| --- | --- | --- |

| (km/h) |  | height of letters |
| --- | --- | --- |
| 0 to 60 | 15 | 150mm |
| 61 to 80 | 12 | 250mm |
| More than 80 | 10 | 350mm |

    - For smaller signs such as estate agent signs, portable signs, banners and flags, posters and advertisements on street furniture, the text size must be a minimum of 50mm high, and must be considered by the Municipality for readability before it can be approved.

  - An advertising sign must have a neat appearance in terms of advertisement content and sign writing, and may not contain untidy handwritten messages. This must be done to the satisfaction of the Municipality.

  - No message may be spread across more than one advertisement, advertising sign or advertising sign panel.

  - Numbers longer than ten digits are not permitted.

  - No advertisement may, in the opinion of the Municipality, be in it's content objectionable, indecent or suggestive of indecency or prejudicial to the public morals; or be in conflict with the guidelines or standards laid down from time to time by the Advertising Standards Authority (ASA), or any similar body recognised as representing the industry.

  - An advertisement may not contain any element which distracts the attention of drivers in a manner likely to lead to unsafe driving conditions.

  - Before any advertising sign is erected, the advertisements must be considered by the Municipality, whether:

    - the size of the advertisement, or any portion thereof by way of its colour, letter size, symbol, logo, graphics or illumination, will result in the advertisement having a distracting effect on the attention of drivers of vehicles to the task of driving and lead to unsafe driving conditions;

    - the colour, or combination of colours, contained in the advertisement correspond with the colours or combinations of colours specified for road traffic signs in the regulations promulgated under the National Road Traffic Act, 1996 (Act No. 1993 of 1996);

    - the portrayal of a road traffic sign in the content of an advertisement will constitute a road safety hazard and could be mistaken to represent a road traffic sign.

# Road safety and traffic considerations

  - No advertisement, advertising structure or sign may

    - in the opinion of the roads authority, be so placed as to distract the attention of drivers or pedestrians in a manner likely to lead to unsafe driving conditions;

    - be so placed, as to cause any obstruction to a motorist's view of the roadway or its approaches, regardless of the direction the motorist is travelling;

    - be so attached as, unless specifically provided for in the Southern African Development Community Road Traffic Signs Manual (SADC RTSM), to obscure, create confusion with or interfere with the functioning of a road traffic sign or signal or create a road safety hazard in the opinion of the roads authority;

    - in the opinion of the roads authority, obscure a pedestrian's or driver's view of pedestrians, road or rail vehicles and features of the road, railway or pavement such as junctions, bends and changes in width;

    - be erected closer than a minimum of 0,5m from the edge of the advertising sign to the vertically projected edge of the shoulder of the road; and 0,3m from a cycle path, footpath or sidewalk;

    - project over a sidewalk or pedestrian route, unless the clear height of such an advertising sign exceeds 2,4m and for a cycle route a clear height of 3,0m;

    - overhang onto a roadway or shoulder (if not on a bridge or a gantry);

    - project or overhang an adjacent erf boundary, except where specific consent has been granted by the relevant land owner, or over any road reserve boundary (excluding projecting signs, flat signs and permanent flags);

be located directly in front of, or behind a road traffic signal or in the opinion of the roads authority, constitute a road safety hazard.

  - Before any advertising sign is erected, it must be considered by the Municipality, whether:

    - the number and size of advertisements and advertising signs in the area, will enhance clutter or affect the impact and effectiveness of road traffic signs by virtue of potential visual clutter;

    - the speed limit, and the measure of the traffic's adherence thereto, the traffic volume, the average following headway and accident history of the road demand more stringent control of outdoor advertising;

    - the position of the advertisement or advertising sign will negatively affect the visibility of, sight distance to or efficiency of any road traffic sign, or series of such signs;

    - the position of any advertisement or advertising sign would potentially distract drivers' attention at places where traffic turns, negotiates curves, merges or diverges, or in the area of intersections or interchanges, or where drivers' uninterrupted attention to the driving task is important for road safety;

    - the distance between advertising signs where the advertisement and the content of the road sign are visible from the same direction of travel before, between or behind any road traffic sign is of such a nature as to distract a driver's attention from any road traffic sign.

- Areas of control

  - Three areas of control apply - areas of maximum control, areas of partial control, and areas of minimum control. These areas of control address the potential interaction between basic landscape sensitivity and advertising sign impact.

  - All areas under the jurisdiction of the Municipality shall be classified under a specific area of control. If an area has not be designated, for whatever reason, it will be deemed to be an area of maximum control, until considered and otherwise classified by the Municipality.

  - The land use categories described below are generic and do not refer to a specific Town Planning Scheme. When an assessment is evaluated, the Town Planning Scheme or any other relevant document applicable to the erf in question will be consulted for zoning details, if available. Notwithstanding the designation of the areas of control, the Municipality may approve the erection of an advertising sign or advertisement if there is no major impact on surrounding areas, and if the surrounding land uses can accommodate that specific type of sign. In instances where the area of control is unclear, the legal activities on the property as well as the surrounding areas, will be used as a basis of determining the areas of control.

  - The following types of land uses can be classified under the areas of control, as listed below:

    - Maximum Control

Game reserves Nature reserves Scenic corridors Scenic landscapes

Agricultural land / Farm land Rural smallholdings

Rural-residential areas

Un-proclaimed township area

Conservation areas and natural features inside the urban edge Undeveloped open spaces

Scenic features and areas Historical and architectural sites All residential areas

Plots and urban smallholdings (which are proclaimed) Home-undertakings within residential areas Specifically proclaimed heritage areas and buildings Cemeteries

    - Partial Control

Commercial enclaves, shopping centres and office blocks in residential areas

Commercial ribbon development

Educational facilities, Schools, Universities, Technicons and Colleges Sports fields or stadiums

Rural-commercial areas

    - Minimum Control

Commercial areas Office blocks Shopping centres Industrial areas

Transport nodes (such as taxi and bus ranks, railway stations, etc.)

    - If required, for whatever reason, the surrounding area, adjacent properties, or the current legal use may provide guidance when determining the area of control.

# Position and spacing

  - The following criteria as included in Table 1 will apply on these 'higher impact and larger type' of signs, unless otherwise specified, for position and spacing requirements between any form or combination of any two of the following advertising signs or advertisements, namely:

Billboards, gantries, non-locality bound flat signs, and all other relevant advertising signs as required by the Municipality.

# Table 1: Position & Spacing Requirements

| Speed |  | Distances required between advertising signs visible from each other, measured from the nearest pole of the advertising signs | Distances required between advertising signs and road signs | Distance required from |
| --- | --- | --- | --- | --- |
|  |  |  | where the advertisement and content of the road sign are visible for the same direction of travel measured from the nearest pole of the advertising sign to the road sign, parallel along the road where the sign is located | advertising signs to intersection measured from the nearest pole of the advertising sign to the centre of the cross road, parallel along the road where the sign is located |
| 0 s 60 |  | 100m | 50m | 50m |
| 61 - | 80 | 200m | 100m | 100m |
| > 80 |  | 250m | 200m | 200m |
| Spacing Requirements: Electronic Billboards |  |  |  |  |
| 0 s 60 |  | 1,5km apart | 50m | 200m |
| 61 - | 80 | 1,5km apart | 150m | 200m |

All distances will be measured from the closest edge of the advertising sign to the relevant point.

300m

200m

2km apart

> 80

  - A single sided advertising sign must be displayed perpendicular to or at an angle that is easy readable for the oncoming traffic. In the case of an advertising sign comprising of more than one side the advertisement must be displayed with the axis of symmetry perpendicular to the direction of the oncoming traffic.

# Exempted signs

  - The following advertising signs or advertisements are exempt from the provisions of these By-laws:

    - Any advertising sign displayed inside a sports stadium which is not visible from outside the stadium or not visible from any public road;

    - Any advertising sign which is not aimed at road users and not visible from a public road;

    - Any advertising sign which is displayed inside a building at a distance of more than two metres from any window or other external opening through which it may be seen from outside the building and which is not aimed primarily at attracting the attention of the road user;

    - Any national flag of any country hoisted on a suitable flag pole provided that no advertisement or subject matter is added to the design of the flag or flagstaff;

    - Any banner or flag carried through the streets as part of a procession;

    - Any advertising signs or advertisements which are permanently painted on or attached to the window-glass of a building;

    - Any advertising sign which is incorporated in and forming an integral part of the fabric of a building applicable mostly to historical buildings only if the building or structure is not primarily used for advertising.

# Prohibited signs and areas

No person may erect or display any of the following advertising signs or advertisements or cause or permit any such advertising sign or advertisement to be erected or displayed:

- Against water towers, reservoir or silos, or painted on bridges, water tower, reservoir or silo;

- Any advertising sign, advertisement, flyposter or poster pasted, painted, placed on or against or attached to or otherwise supported by any power line standard, power masts, transformer box, telegraph pole, road traffic sign or signal (any lamp pole which has a road traffic sign or signal attached to it), traffic circle, traffic island or median (except if allowed for in these By-laws or if approved by the Municipality), wall, column or post or pillar of a veranda or balcony, fencing, electricity box or sub-stations, tree, electrical

transmission pylon or bridge or any other municipal infrastructure where specific provision has not been made;

- With an overhang onto a roadway or shoulder, if not on a bridge or a gantry;

- Which will obscure a road traffic sign or which may be mistaken for or cause confusion with or interfere with the functioning of a road traffic sign; or which may obstruct pedestrian traffic;

- Which will obstruct any window or opening provided for the ventilation of a building or

which obstructs any stairway or doorway of other means of exit from a building or which will prevent the movement of persons from one part of a roof to another part thereof;

- Any animated or flashing sign where the frequency or the animation's or flashes or other intermittent alterations of which disturbs the residents or occupants of any building or is a source of nuisance to the public;

- Larger than 81m2 or higher than 12,5m, if free-standing; or larger than 18m2 if in electronic format;

- Any swinging sign, which is a sign not rigidly and permanently fixed;

- Any moveable or transit sign or trailer advertising sign, whether stationery or mobile;

U)		On land not in accordance with the relevant zoning or approved consent use as per applicable Town Planning Scheme or any other relevant document;

- Pasted otherwise than on an advertising structure legally erected for the purpose of accommodating such an advertisement;

  - Which, in the reasonable opinion of the Municipality, is suggestive of anything indecent or immoral or constitutes hate speech;

- Which relates to a business which is conducted on an erf or land which has not been re­ zoned and approved for that specific purpose;

- Which relates to cigarettes, tobacco products or any other advertisements prohibited by any official advertising authority;

- Banners suspended across a road, affixed to a bridge spanning a road, placed on or against or attached to or otherwise supported by any transformer box, telegraph pole, tree, road traffic sign, electric light standard or other sign or object;

- For any estate agent sign containing any other words except the words 'For Sale', To

Let', 'Sold' or 'On Show' and the name, logo, address and telephone number of the selling agent or letting agent;

- Other than an exempted sign, for which no approval has been obtained;

- Any sign which does not comply with the requirements of a specific type of sign as prescribed in these By-laws, for which no previous approval exist.

PART	C:		CRITERIA	FOR	DIFFERENT	TYPES	OF	ADVERTISING	SIGNS	AND ADVERTISEMENTS

# Billboards

  - The following criteria as included in Table 2 will apply for billboards and any other advertising signs, as prescribed:

Table 2: Location, Size and Height

* Super Billboards (only with special consent)

Large Billboards

Small Billboards and Electronic signs

Area of control	Minimum	Minimum / Partial	Minimum / Partial

Size of advertisement

Speed (where permitted)

Maximum Height of advertising sign

Larger than 40mL up to and including 81m2

Any speed

12,5m

Larger than 18mL up	Larger than 4,5mL up to to and including 40m2		and including 18m2

Only on roads with	 Only on roads with speed limits less than	speed limits less than or equal to 80 km/h	or equal to 80 km/h

10,5m	10,5m

    - No billboard shall exceed the allowable height on that specific site or on the surrounding sites, as allowed for in the relevant Town Planning Scheme or any other relevant document, or approved zoning for the site, whichever is the lesser, unless specifically approved by the Municipality.

    - * Super Billboards larger than 40m2 up to and including 81m2 will only be allowed if special attention and special consent has been given by the relevant Local Municipality, in exceptional cases. It will only be considered if the environment can accommodate it and if the market will allow it, and will only be allowed in urban areas of minimum control.

    - Billboards may be permitted inside municipal road reserve or other Municipal owned land, with specific attention to:

      - Road Safety (Section 19),

      - Combination and clutter of advertising,

      - Environmental impact.

  - The clear height of the advertising structure may not be less than 2,4m.

  - Criteria as included in Table 1 above, applies.

  - Illumination is permitted subject to Section 17. No animation is permitted, except for electronic billboards.

  - General requirements, as stated in Section 14 to 21 apply.

  - No billboard may be erected within a radius of 50m from the edge of any area of maximum control, measured from the nearest edge of the advertising sign.

  - Community awareness billboards are limited to a maximum size of 6m2 in urban areas of maximum control, and limited to 18m2 (small billboard) in all other areas of control. Criteria

as included in Table 1 and Table 2 above, applies. It will be allowed in all urban areas of control and in rural-residential areas, but not in rural or natural areas of control.

  - In a rural-commercial areas, billboards are limited to a maximum size of 18m2.

  - If positioned in the Municipal road reserve, the horizontal clearance of the nearest footing or the upright of the advertising structure must be a minimum of 5m from the shoulder breakpoint of the road. In the event of a kerbed road, 4,5m will be permitted. The horizontal clearance of the footing or the upright may be reduced to 2,5m, and must then be protected by guard rails.

  - This class is subject to the approval of the Municipality.

# Gantry signs

  - This class will only be permitted in urban areas of minimum control.

  - The size and height of advertising signs allowed in this class, are as follows:

    - Size of advertisement: 36m2 to 81m2. Gantries larger than 40m2 will only be allowed if special attention and special consent has been given by the relevant Local Municipality, in exceptional cases. It will only be considered if the environment can accommodate it and if the market will allow it.

    - Maximum height from the road surface to the top of the advertising sign: 12,5m.

    - Minimum clear height of the advertising sign from the highest point of the road surface should be 5,7m; except in the event where the Municipality may prescribe additional clearance.

  - Criteria as included in Table 1 above apply, and in addition gantries must be spaced at least 1km apart on roads with a speed limit of s; 80kph if visible from each other.

  - Illumination is permitted subject to Section 17. No animation is permitted, except for electronic billboards.

  - General requirements as in Section 14 to 21 apply.

  - No gantry may be erected within a radius of 50m from the edge of any area of maximum control, measured from the nearest edge of the advertising sign. Should the uprights of a gantry be positioned adjacent to areas of minimum and partial control respectively, the area of control will be classified as minimum control.

  - The horizontal clearance of the nearest footing or the upright of the advertising structure must be a minimum of 5m from the shoulder breakpoint of the road. In the event of a

kerbed road, 4,5m will be permitted. The horizontal clearance of the footing or the upright may be reduced to 2,5m, and must then be protected by guard rails.

  - This class of advertising sign as well as the design of the advertising structure is subject to the approval of the Municipality.

- Roof signs

  - Roof signs will be permitted in areas of partial and minimum control.

  - Only locality-bound roof signs shall be permitted.

  - The size and height of advertising signs allowed in this class, are as follows:

    - Size of advertisement - Maximum area: 18m2

    - Height: May not be higher than the height restriction in terms of the relevant Town Planning Scheme or any other relevant document.

  - The position and spacing requirements for this class of advertising signs are as follows:

    - Maximum one advertising sign per building.

    - Not project in front of a main wall of host building.

    - Bottom of sign not more than 120mm above roof.

  - These advertising signs may be illuminated, subject to Section 17, but not animated.

  - A roof sign shall not exceed 300mm in thickness.

  - This class consists of advertising signs which are fixed to the roofs of buildings used or partly used for commercial, office, industrial or entertainment purposes, it may also include any advertising sign consisting of a single line of free-standing, individual, cut-out, silhouetted letters, symbols or emblems.

  - General requirements as stated in Section 14 to 21 apply.

  - No roof sign may be erected within a radius of 50m from the edge of any area of maximum control, measured from the nearest edge of the advertising sign.

  - This class is subject to the approval of the Municipality.

- Flat signs

  - Flat signs will be permitted in rural areas and all urban areas. In areas of maximum control, only locality-bound signs will be permitted.

  - The size of advertising signs allowed in this class, are as follows:

    - Locality-bound:

Rural and urban areas of maximum control: Not larger than 18m2 or a maximum area of 50% of the wall area, whichever is the lesser. The sponsor's name or logo may not contain more than 1/3 of the total advertisement area.

Urban areas of partial and minimum control: Not larger than 40m2 or a maximum area of 70% of the wall area, whichever is the lesser. The sponsor's name or logo may not contain more than 1/3 of the total advertisement area.

    - Non-locality bound:

Only in urban areas of partial and minimum control. Not larger than 40m2 or a maximum area of 70% of the wall area, whichever is the lesser, spaced according to Table 1 (Section 21) if the advertisement is simultaneously visible by travelling motorists.

  - The position and spacing requirements and some general conditions for this class of advertising signs are as follows:

    - Maximum control: One per business per street frontage.

    - Partial & minimum control: Two per business per street frontage.

    - Not to obstruct or conceal any windows or the view or light from such windows or any opening provided for the ventilation.

    - Not to extend above top/ beyond either end of wall.

    - May not project more than 50mm if less than 2,4m above sidewalk or ground level or 300mm where an advertising sign is more than 2,4m above the sidewalk or ground level.

    - Where locality-bound and non-locality bound advertisement are located on one building, it may not be combined, constructed or affixed in any manner on one wall.

  - These advertising signs may be illuminated, including non-locality bound flat signs which may be electronic signs, subject to Section 17, but not animated.

  - This class consists of advertising signs which are affixed to any external or main wall of a building used for commercial, office, industrial or entertainment purposes, excluding a parapet wall, balustrade or railing of a veranda or balcony of such a building. It may consist of a panel/sheet or of individual numbers, letters or symbols.

  - General requirements as stated in Section 14 to 21 apply.

  - This advertising sign type shall not be applicable to buildings used for residential purposes or for community services or community institutions, small businesses and practices on residential premises, or small scale residential-oriented accommodation.

  - No non-locality bound flat sign may be erected within a radius of 50m from the edge of any area of maximum control, measured from the nearest edge of the advertising sign.

  - It must not obstruct any window or opening provided for the ventilation of a building or which obstructs any stairway or doorway of other means of exit from a building.

  - This class is subject to the approval of the Municipality.

- Advertising signs at educational facilities and sport stadiums and fields

  - Advertising signs at educational facilities and sport stadiums and fields will be permitted in all areas of control.

  - The size and height of the sponsored advertising signs indicating the name of the facility, are as follows:

    - Size:	Maximum of 12m2.

    - Height:	Maximum of 5,5m.

    - Clear height: 2,4m.

    - Sponsor name or logo: Maximum of 1/3 of total advertisement area.

  - A maximum advertisement area of 36m2, per 100m of street front, may be permitted for the display or combination of advertising signs on boundary walls and fences, permanent flags and free-standing advertising signs smaller than 4,5m2.

  - Any form of advertising sign or advertisement may be displayed, subject to the criteria and requirements for that specific type of advertising sign or advertisement, with specific attention to cluttering.

  - Only one sponsored advertising signs indicating the name may be permitted per street frontage.

  - Illumination is permitted in areas of minimum and partial control subject to Section 17, while only limited external illumination may be permitted in areas of maximum control, but not animation.

  - This class consists of the display of advertising signs on boundary walls and fences, permanent flags, equal sized free-standing advertising signs smaller than 4,5m2, other free­ standing advertisements and may include a sponsored advertising sign indicating the name, along the street frontage of educational facilities and sport stadiums and fields.

  - General requirements as stated in Section 14 to 21 apply.

  - This class is subject to the approval of the Municipality.

# Service facility signs

  - Service facility signs will be permitted in all areas of control.

  - The size and height of advertising signs allowed in this class, are as follows:

| J Speed of the road (km/h) | Maximum Height | Maximum Width |
| --- | --- | --- |
| 0 up to and including 60 | 7m | 2m |
| 61 up to and including 80 | 10m | 3m |
| More than 80 | 15m | 6m |

  - A maximum of eight advertisement panels may be permitted per service facility sign and only one business or service may be permitted per advertisement panel.

  - The position and spacing requirements for this class of advertising signs are as follows:

    - Only at service facilities adjacent and directly accessible from road where sign is.

    - One combination sign per site which may be double-sided.

    - Located according to requirements of the relevant road authority.

    - Not on road median or on island.

    - If the advertising sign cannot be located on the site and has to be located in the road reserve, specific approval must be obtained both from the Municipality and the relevant road authority. It should then be located as close as possible to the access and in front of the service facility site.

  - These advertising signs may be illuminated, subject to Section 17, and only if the facility is open 24 hours or during business hours of the specific service. No animation is permitted.

  - This class consists of combination signs displayed at filling stations and roadside service areas (rest and service areas) which may provide a variety of services such as fuel pumps, workshops, restrooms, car washes, shops, accommodation facilities, restaurants, fast food outlets and autotellers.

  - General requirements as stated in Section 14 to 21 apply.

  - Advertisements on such combination signs may refer only to the name and logo of a business, company or person providing a service or may indicate the type of service provided. Only advertisements for locality-bound services shall be permitted.

  - Supplementary signs at roadside service areas, which do not form part of a combination sign permitted under this class, must be used for internal direction and orientation only and may not be aimed at passing motorists.

  - This class is subject to the approval of the Municipality, except locality-bound canopy, internal direction and orientation signs at filling stations and road side rest and service areas.

# On-premises business signs

  - On-premise business signs will be permitted in the following areas of control:

    - Individual advertising signs:	All areas of control.

    - Combination signs:	Partial and minimum control.

  - The size and height of individual advertising signs allowed in this class, are as follows:

    - Size: Maximum control:

Partial / Minimum control:

    - Height:

    - Sponsor name or logo:

Maximum area: 6m2. Maximum area: 12m2.

Maximum 7,5m (may be increased to 10,5m if permitted in the Town Planning Scheme or any other relevant document).

Maximum of 1/3 of total advertisement area.

  - The size and height of combination signs, are as follows:

    - Size: Maximum 16m2.

Maximum size may be increased to 23m2 if height restriction is relaxed in terms of the Town Planning Scheme or any other relevant document.

Minimum dimension per panel: 1m x 1m.

Not less than four and not more than ten panels per side will be permitted.

    - Height:

Maximum 7,5m.

Height may be increased to 10,5m if permitted in the Town Planning Scheme or any other relevant document.

    - Width of structure:

Not less than 2,1m and not more than 3m.

    - Clear Height: 2,4m.

  - The position and spacing requirements for these advertising signs are as follows:

    - Only locality-bound advertisements.

    - Individual free-standing advertising signs will only be permitted if it is not possible to affixed on-premises signs to the building, or if signs affixed to the building will not be noticed by passing pedestrians or motorists, or if the sign is needed to located the entrance to the business.

    - The relevant Consent use or Town Planning Scheme or any other relevant document, is applicable.

    - One sign per street frontage (max two per site).

    - Placed close to business or if not close or visible from road, then at entrance road.

  - Preference will be given to combination signs. Combination signs may incorporate several businesses on one sign, using a single panel per business. In the case of a combination sign, the message on individual panels of the sign must be concise and legible, signs must harmonious in terms of form, letter types and colour, the signs shall be designed and located so as not to create a traffic safety hazard.

  - Where a business or enterprise such as a stall or guest house is situated on a large property such as a farm the sign shall be placed in the immediate vicinity of the enterprise where such an enterprise is adjacent to or visible from a public road; if the enterprise is not adjacent to or visible from a public road the sign shall be placed at the entrance of the private access road to the enterprise.

  - Signs indicating roadside enterprises such as farm stalls, and businesses on farms or roadside cafes shall not be closer than 5m from the road reserve fence. Such enterprises shall have direct access to the public road.

  - Illumination is permitted in areas of minimum and partial control subject to Section 17, while only limited external illumination may be permitted in areas of maximum control, but may not be animated.

  - This class consists of free-standing advertising signs with locality-bound advertisements which are aimed at identifying and locating the business operating on that site for businesses, industries, farm stalls and businesses on farms and smallholdings.

  - Advertisements in this class shall refer only to the name, logo and nature of the business on the premises, the brand-name and nature of goods and products sold, the nature of services provided, the name of the proprietor(s) or practitioner(s) and a contact number.

  - General requirements as stated in Section 14 to 21 apply.

  - This class is subject to the approval of the Municipality.

# Projecting Signs

  - Projecting signs will be permitted only in urban areas of minimum, partial and maximum control, but shall be limited to buildings utilized for commercial, industrial, or entertainment purposes.

  - The size and height of advertising signs allowed in this class, are as follows:

    - Size: Maximum control:

.

Size: Partial /Minimum control:

If below 6m: maximum 1,2m2. If above 6m: maximum 4m2. If below 6m: maximum 2,4m2. If above 6m: maximum 8m2

  - The position and spacing requirements for this class of advertising signs are as follows:

    - Only locality-bound advertisements.

    - Only one per business fa9ade.

    - At an angle of 90° to the direction of oncoming traffic.

    - Horizontal distance between the edge of the advertising sign to the vertically projected edge of the shoulder: More than 0,5m.

    - May not project more than 300mm from the surface of the main wall.

  - These advertising signs may be illuminated, subject to Section 17, but not animated.

  - This class consists of advertising signs which are affixed to an external wall of a building used for commercial, office, industrial or entertainment purposes.

  - General requirements as stated in Section 14 to 21 apply.

  - This advertising sign type shall not be applicable to buildings used for residential purposes or for community services of community institutions, small businesses and practices on residential premises, or small-scale residential-oriented accommodation.

  - A projecting sign may encroach on a sidewalk. It may not be fixed at a clear height of less than 2,4m and for a cycle path a clear height of 3,0m nor exceed 300mm in thickness.

  - A projecting sign may not extend beyond the top of any wall.

  - This class is subject to the approval of the Municipality, only if it overhangs Municipal land.

# Advertising signs on boundary walls and fences

  - Advertising signs on boundary walls and fences will be permitted in all areas of control. In areas of maximum control, only locality-bound signs will be permitted.

  - The size of the advertising signs allowed in this class, are as follows:

    - Locality-bound advertisement:

Maximum control:

Partial / Minimum control:

Maximum advertisement area 6m2 Maximum advertisement area 12m2

The relevant Consent use, or Town Planning Scheme, or any other relevant document, is applicable.

Sponsor name or logo:

    - Non-locality bound advertisement:

Maximum of 1/3 of total advertisement area.

Maximum advertisement area per 100m streetfront: 36m2 Advertising signs are to be of equal size.

  - The position and spacing requirements for this class of advertising signs are as follows:

    - Only one locality-bound advertising sign per street frontage per premises.

    - Non-locality bound advertising signs: Minimum spacing of 5 m intervals.

    - It may not extend above, below or beyond any extremity of the boundary wall or fence.

  - This class of signs may be illuminated, subject to Section 17, but not animated.

  - This class consists of advertising signs affixed to a boundary wall or fence.

  - General requirements as stated in Section 14 to 21 apply.

  - This class is subject to the approval of the Municipality.

# Painted advertisements

  - Painted advertisements will be permitted in areas of partial and minimum control, and in rural-residential areas.

  - Only locality-bound painted signs will be allowed.

  - The position and spacing requirements for this class of advertisements are as follows:

    - One advertisement per business and if on the facade of the building, then one per street frontage.

    - Permitted on the fac,;:ade of a building, on the walls and roofs of the specific enterprise.

  - Painted walls as advertisements may also be allowed in the road reserve, if placed in a safe position, not in the view of, or interfering with sight distances or safety of motorists, and approved by the relevant road authority. It may not be located within a 50m radius from the centre of any intersection. Painted walls in the road reserve is limited to a maximum size of

4.5m2 and a maximum height of 2m.

  - These advertisements may not be illuminated or animated.

  - This class consists of advertisements painted directly on the main walls or roofs of a building used for commercial, office, industrial or entertainment purposes, or on boundary walls of a business or home-business.

  - General requirements as stated in Section 14 to 21 apply.

- Security advertising signs

  - Security advertising signs will be permitted in all areas of control.

  - The size and height of advertising signs allowed in this class, are as follows:

    - Size:	Security services signs:	Maximum area: 0,35m2 Neighbourhood and farm watch:	Maximum area 1,5m2

    - Height:	Maximum 3m

  - The position and spacing requirements for this class of signs are as follows:

    - Security services signs:

- One advertising sign per street frontage firmly affixed to the building, boundary wall, fence or gates or on the stand.

- Minimum spacing between signs: 30m length of street boundary.

    - Farm watch advertising signs:

- At the junction or intersection of a public road and private access road or at the entrance to an individual farm.

- Only one advertising sign per farm may be permitted.

    - Neighbourhood watch advertising signs:

(i)  Within a municipal road reserve, at the points where the watch areas are entered.

    - Not on a road island or median.

  - These advertising signs may not be illuminated or animated.

  - This class consists of an advertising sign which may only refer to the existence and operation of neighbourhood watch, commercial security service, burglar alarm system, farm watch and similar watch schemes.

  - General requirements as stated in Section 14 to 21 apply.

- Balcony, veranda, canopy and under-awning signs

  - Veranda, balcony, canopy and under-awning advertising signs will be permitted in all areas of control, but only on premises or buildings used for commercial, office, industrial or entertainment purposes.

  - Only locality-bound signs will be allowed.

  - Clear height of signs must be 2,4m and may not project more than 100mm from the surface to which it is affixed.

  - The size, position and spacing requirements for this class of signs are as follows:

    - May not exceed a vertical dimension of 750mm and a horizontal dimension of 2400mm.

    - One sign per business or enterprise.

    - May be suspended above sidewalks.

    - May not extend above, below or beyond any extremity or a parapet wall, balustrade, railing, beam or fascia.

  - These signs may be illuminated, subject to Section 17, but not animated.

  - This class consists of balcony, veranda, canopy and under awning signs which are:

    - Affixed flat onto or painted on a parapet wall, balustrade or railing of a verandah or balcony;

    - Affixed flat onto or painted on a fascia of a verandah or beam over verandah columns;

    - Affixed flat onto or painted on the fascia of a roof structure without walls such as a roof covering petrol pumps at a service facility (filling) station;

    - Suspended below the roof of a veranda or balcony (under awning signs);

    - Placed on top of a roof or veranda;

    - Affixed to or painted on a pillar, column or post supporting a veranda, balcony or a roof structure without walls;

    - Painted or printed on the fabric of a blind.

  - General requirements as stated in Section 14 to 21 apply.

# Residential home-undertaking, farm names and community institution signs

  - Residential home-undertaking, farm names and community institution signs will be permitted in all areas of control for home-undertakings, names of farms and community institutions.

  - The size and height of advertising signs allowed in this class, are as follows:

    - The size of the home-undertaking or farm name sign may not exceed 1,5m2.

    - The size of the community institutions sign may not exceed 3m2.

    - The height may not exceed 3m for free-standing signs.

    - The relevant Consent use, or Town Planning Scheme, or any other relevant document is applicable.

    - Sponsor name or logo: Maximum of 1/3 of total advertisement area.

  - The position and spacing requirements for this class of advertising signs are as follows:

    - Only locality-bound advertisements on boundary wall, fence, gates.

    - Farm/small holdings name signs: Next to entrance of access road or on gate of entrance.

    - Free-standing only when not possible to fix to building / wall / gate / boundary fence limited to one sign only.

    - One sign per undertaking, farm or institution per street frontage.

    - Only locality-bound advertisements.

    - Not in road reserve.

  - These advertising signs may be illuminated, subject to Section 17, except advertising signs in natural and rural areas of control. These advertising signs may not be animated.

  - This class consists of signs for small business and practices on urban residential premises or in rural areas and farm names, and includes signs for community institutions and facilities such as religious, cultural, recreational and certain medical and similar institutions.

  - General requirements as stated in Section 14 to 21 apply.

  - The sign may only indicate the name, logo and nature of the undertaking or institution or the farm name and no product advertising or advertising for sales of any kind are permitted on this advertising sign.

- Vehicular advertising

  - Vehicular advertising will be permitted in all areas of control.

  - These advertising signs may not extent beyond the edges of the vehicle.

  - The vehicle may not be parked in the road reserve with the sole purpose of advertising.

  - These advertisements may not be illumination or animated.

  - This class consists of advertisements on self-driven vehicles which are normally moving on land or water, including taxis, buses, trains and delivery vehicles.

  - General requirements as stated in Section 14 to 21 apply.

- Construction site boundary signs

  - Construction site signs will be permitted in all areas of control.

  - Construction site boundary signs may not exceed a vertical dimension of 3,0 metres and a total area of advertisement of 18m2.

  - The position and spacing requirements for this class of advertising signs are as follows:

    - Not on top of or above the fence or wall.

    - Not project more than 0,1m from boundary wall or fence.

    - May not be painted/pasted directly on construction site boundary wall.

    - May not encroach on the road reserve unless approved by the Municipality.

  - These advertising signs may be externally illuminated from the top, subject to Section 17, but may not be animated. Illumination will not be permitted where it will impact on areas of maximum control.

  - The class consists of advertising signs fixed flat against any fence or wall where such fence or wall forms the boundary of a site where construction work is being carried out, or affixed to walls of unoccupied buildings undergoing construction, renovation or maintenance.

  - General requirements as stated in Section 14 to 21 apply.

  - It will only be permitted for the duration of the construction, renovation or maintenance work whichever is the shorter.

  - This class is subject to the approval of the Municipality.

# Project and development signs

  - Project and development advertising signs will be permitted in all areas of control.

  - The size and height of advertising signs allowed in this class, are as follows:

    - Project sign:

- Maximum size: 1,5m2 per consultant.

- Maximum size: 9m2.

- Height: Maximum 5,5m.

- Clear Height: 2,4m.

    - Development sign:

- Maximum size: 6m2 in maximum control area.

- Maximum size: 12m2 in partial and minimum control area.

- Height: Maximum 5,5m.

- Clear Height: 2,4m.

  - The position and spacing requirements for this class of advertising signs are as follows:

    - Project sign:

- One sign per street-front on the specific site.

- Not in road reserve.

- Only road construction signs will be permitted within the road reserve.

    - Development sign:

- Only one development sign per street frontage per development on the specific site.

- Only one additional sign with supplier detail is permitted per development.

- Signs indicating supplier details may only be erected within the site and may not be positioned where intended for or visible by passing vehicle or pedestrian traffic.

- Not in road reserve.

  - The general requirements as stated in Section 14 to 21 apply.

  - Project signs consist of advertising signs displaying the involvement of contractors and consultants in minor or major construction projects or alterations to existing structures or facilities and the development sign describes the type of development.

  - Requirements applicable to PROJECT SIGNS:

    - Project signs may not be illuminated or animated.

    - The advertisement shall describe only the building or structure being erected or other work or activity being carried out during the duration of the project, and the names of the contractors or consultants concerned in such work or activity. The branches of the industry or the professions of the contractors or consultants may be listed.

    - Individual or single signs must be displayed only if no other consultants or contractors are involved or if a combined project sign has already been erected.

    - Only one advertisement or advertising sign per contractor or consultant per project may be permitted.

    - Project sign concerning road construction may be positioned in any road reserve.

    - Project sign must be displayed only during the period when the construction works are actually taking place on the site.

  - Requirements applicable to DEVELOPMENT SIGNS:

    - Included are advertisements describing the type of development being carried out on a site and giving details such as the type of accommodation being provided, floor space available and the name, address and telephone number of the developer or his agent.

    - A visual presentation or description of the building or structure being erected may also be included.

    - Advertising signs or advertisements indicating supplier details are to be combined on only one sign per development and restricted to a maximum of nine uniform panels on the sign.

    - Development advertising sign may be illuminated, subject to Section 17, but may not be animated.

    - Only while relevant development is taking place.

  - This class is subject to the approval of the Municipality.

# Advertisement for the sale of goods and livestock

  - This class will be allowed in all areas of control.

  - The size and height of signs allowed in this class are as follows: Size:	Maximum or Partial Control:

Minimum Control: Maximum height:

  - The position and spacing requirements for this class of signs are as follows:

    - Not on road reserve or road reserve boundary of freeways.

    - Maximum of one sign per sale facing a road.

    - Only on the premises, on the property, or attached to boundary fence of property.

    - May be displayed 14 days prior to the event and should be taken down within 3 days after the event.

  - These signs may not be illuminated or animated.

  - This class consists of signs announcing the sale of goods or livestock on land or on premises not normally used for commercial purposes. It may include auction sales of furniture and other household goods on residential premises or an auction of livestock or game on a farm.

  - General requirements, as stated in Sections 14 to 21, apply.

# Estate agent signs

  - Estate agent signs will be permitted in all areas of control.

  - The size and height of advertising signs allowed in this class, are as follows:

    - Maximum size: Non-residential, commercial or industrial signs: 6m2.

    - Maximum Height: Non-residential, commercial or industrial signs: 3m.

    - Size: All other signs: 0.6m x 0.45m.

  - The position and spacing requirements for this class of advertising signs are as follows:

    - Placed close to a boundary fence or within boundary of erf.

    - Maximum one advertising sign per agency.

    - Maximum three advertising signs per erf.

    - Signs for a non-residential, commercial or industrial site is not permitted in the road reserve.

  - These advertising signs may not be illuminated or animated.

  - This class consists of advertising signs which are temporarily displayed, by the owner or its agent, to advertise the fact that land, premises, development or any other form of real estate is for sale, to let, sold or on show.

  - General requirements as stated in Section 14 to 21 apply.

  - All advertisements in this class shall contain only the words 'For Sale', To Let', 'Sold' or 'On Show' and the name, logo, address and telephone number of the selling agent or letting agent.

  - The erection of 'On Show' estate agents' signs on streets reserves shall be permitted on public holidays from 05:00 on condition that they be removed before 20:00 on that day and after 12:00 on Fridays on condition that they be removed before 12:00 on the following Monday.

  - An advertising sign may consist of a single sign or two duplicate signs joined together.

  - Any estate agents sign exceeding 0,27m2 is subject to the approval of the Municipality.

  - The advertising signs must be placed at or affixed to the building concerned, or attached to the boundary fence of the premises concerned, or displayed within the boundaries of such premises, or displayed on the sidewalks to a maximum of 1 metre away from the boundaries of such premises.

  - 'On Show' estate agent signs shall be allowed to be displayed within the road reserve in the event of the advertising sign being a residential advertising sign. Advertising signs may not be displayed on road islands or medians. 'On Show' signs may be displayed with no less than 60m intervals between the signs, and no more than 5 signs being displayed at any one time. This includes 'On Show' signs for new developments.

  - All 'For Sale' and To Let' signs must be removed no later than 3 days after completion of the sale or granting of the tenancy.

  - 'Sold' signs may be displayed for a period not exceeding 30 days after completion of the sale.

  - 'On Show' signs for new developments may be erected for a period of six (6) months where after an extension of an additional six (6) months may be obtained at the discretion of the Municipality.

  - No advertisement relating to the sale, show or lease of a fixed property may be displayed in any manner or in view of any street, unless the Agency's prescribed annual license tariff is paid, particulars of the concerned estate agency are recorded on a database, and the necessary prescribed license is issued by the Municipality.

  - This class is subject to the approval of the Municipality.

# Portable or forecourt signs

  - Portable or forecourt advertising signs will be permitted in all areas of control.

  - The size and height of advertising signs allowed in this class, are as follows:

    - Size:

Maximum area:	Single sided: 0.75m2. Double sided: 1.5m2.

Maximum area per forecourt frontage/ premises: 3m2.

For filling stations & roadside service areas:

Maximum area per forecourt frontage/ premises: 8m2.

    - Height:	Maximum 1m.

  - The position and spacing requirements and some general conditions for this class of advertising signs are as follows:

    - In forecourts of businesses or on sidewalk in front of business premises.

    - Free-standing and moveable.

    - Not be positioned in such a way as to interfere with pedestrian movement.

    - Forecourt signs shall be aimed at passing pedestrians and the users of the forecourt space concerned and shall not be aimed at passing motorists.

  - These advertising signs may not be illuminated or animated.

  - This class consists of moveable free-standing temporary advertising signs displayed in forecourts of business and on sidewalks in front of business premises to draw attention to any commercial services, goods for sale, or other services available at the premises.

  - General requirements as stated in Section 14 to 21 apply.

  - A forecourt is an outdoor area which forms a functional part of a building, housing an enterprise and may include a terrace in front of a restaurant or cafe, a sidewalk cafe, the area at a filling station where the pumps are situated, etc.

  - A maximum of one sign is permitted per business only during business hours, and must be removed from the forecourt or sidewalk on close of business at the end of normal trading hours.

  - Hand-written messages are permitted on signs provided for this purpose.

# Banners and flags

  - Banners and flags will be permitted in all areas of control except in natural areas.

  - This class consists of two categories namely, banners and flags:

    - Banners: Banners erected to advertise public and charitable events, functions, occasions, meetings or campaigns of a religious, educational, cultural, political, social, sporting or recreational nature. This includes banners erected to advertise an auction as well as public awareness and community based campaigns and notices of a public meeting. It also includes banners for parliamentary or municipal elections, by­ elections, referenda and registration process.

    - Flags: Permanent and semi-permanent flags displaying the name, logo and nature of the specific business.

  - The size and height of advertising signs allowed in this class, are as follows:

# Banners

Maximum size: 5m2 Height: Maximum 2m

Maximum advertisement area per event per streetfront: 10m2

# Flags

Maximum size: 4m2 Height: Maximum 5m

  - The requirements for banner advertising signs are as follows:

    - Maximum of 5 banners per event.

    - On the site / boundary fences / walls of where the function / event is to take place or on boundary fences / walls of a third party or in positions within the road reserve as determined by the Municipality.

    - Minimum distance from centre of intersection: 50m.

    - Minimum distance from road sign or signal: 50m.

    - Minimum of 120m apart if in road reserve.

    - No banner advertisement may be displayed for more than two weeks before the date of the function or event advertised and no such advertisement shall be permitted to remain in position for more than three days after the conclusion of such function or event.

    - No banner may be suspended across a road, be affixed to a bridge spanning a road, placed on or against or attached to or otherwise supported by any transformer box, telegraph pole, tree, road traffic sign, electric light standard or other sign or object.

    - No banner may be positioned within the roadway or shoulder of a road.

    - Banners for parliamentary or municipal elections, by-elections, referenda and registration process are only permitted to be placed against the boundary walls / fences or on the premises of the voting/registration station.

U) A completed street name list on where the banners will be erected, is to be provided by the applicant, within at least three days prior to the event, occasion, function or meeting as indicated on the banner.

- All banners must be removed within 3 days of the passing of the event. Failure to remove a banner, will result in forfeiting the deposit for that banner.

  - Every deposit paid must be refunded when all the banners to which the deposit relates, have been removed, to the satisfaction of the Municipality.

(m) Any person who, having displayed or caused to be displayed any banner, fails to remove it or cause it to be removed within the periods prescribed shall be guilty of an offence and shall, in addition to any penalty imposed upon him, forfeit the deposit relating to it or such proportionate part of that deposit as the Municipality must access having regard to the number of banners not removed.

  - The requirements for flag advertising signs are as follows:

    - Maximum of 10 permanent flags per street front.

    - Maximum of 4 semi-permanent flags per street front per business premises.

    - Minimum of 5m spacing between any two flags.

    - Flags are to be of equal size.

    - Only locality-bound flags must be used for advertising businesses.

    - Semi-permanent flags may be displayed directly in front of the specific business in the road reserve in positions as determined by the Municipality but may not be positioned within the roadway or shoulder of a road. They must be removed on close of business at the end of normal trading hours.

    - Every permanent flag must be attached to a single flagstaff projecting vertically from premises or projecting vertically, horizontally or at an angle from a building on the site or against the building or boundary walls / fences where the business is located or attached to a free-standing flag staff on approved sites.

    - No permanent flag is permitted to be displayed within the road reserve

    - Flags may only display the name, logo and nature of the business.

U)  The clear height of the permanent flag may not be less than 2,4m.

- If projecting over a sidewalk and pedestrian circulation route, the clear height must exceed 2,4m and for a cycle circulation route a clear height of at least 3,0m

  - No person may display or erect any flag which relates to a business which is conducted on an erf or land which has not been re-zoned for that specific purpose.

(m) No flag shall be displayed in any manner or in view of any street, unless the prescribed annual license tariff is paid, particulars of the concerned business is recorded on a database, and the necessary prescribed license is issued by the Municipality.

  - The general requirements for this class of signs are as follows:

    - These advertising signs may not be illuminated or animated.

    - General requirements as stated in Section 14 to 21 apply.

    - Banners and flags may not be used for advertising sales promotions or commercial products or events.

    - Every banner or flag must be attached so as not to interfere with or constitute a danger to passing vehicular or pedestrian traffic and may not be displayed within or across any footpaths. Sidewalks or cycle circulation routes.

  - National flags of any country are excluded from this class and may therefore be displayed in all areas of control provided they do not carry any advertisement or subject matter additional to the design of the flag or flagstaff.

  - This class is subject to the approval of the Municipality.

- Posters

  - Posters will be permitted in areas of maximum, partial and minimum control.

  - This class consists of five categories of posters namely:

Category One: Events: Posters erected to advertise public and charitable events, functions, occasions, meetings or campaigns of a religious, educational, cultural, political, social, sporting or recreational nature. This category includes posters erected to advertise an auction but excludes posters advertising sales promotions, commercial products and events of a commercial nature.

Category Two: Commercial: Posters erected in this category may be used for commercial advertising on structures for which specific provision has been made for by the Municipality. Category Three: Newspaper: Posters displayed in this category must display selected news headlines of a specific edition of a newspaper.

Category Four: Community: Posters for public awareness and community based campaigns, notices of a public meeting.

Category Five: Elections: Posters for parliamentary or municipal elections, by-elections, referenda and registration process.

  - The size and height of advertising signs allowed in this class, are as follows:

CATEGORY ONE (Events) AND FIVE (Elections):

Size:	One direction:  s 0.54m2, 0.9m x 0.6m (A1 size), may be double sided.

Height: Minimum Height (Clearance): 2,4m. At least 2m below light fixtures.

CATEGORY TWO (Commercial):

Size:	One direction: s 1.08m2, 0.2m x 0.9m (AO size), may be double sided. Height: Minimum Height (Clearance): 2,4m. At least 2m below light fixtures.

CATEGORY THREE (Newspapers):

Size:	One direction: s 0.32m2, 0.7m x 0.45m (A2 size), may be double sided. Height: Minimum Height (Clearance): 1m. At least 2m below light fixtures.

CATEGORY FOUR (Community):

Size:	One direction:	s 1.08m2, 1.2m x 0.9m (AO size), may be double sided. Height: Minimum Height (Clearance): 2,4m. At least 2m below light fixtures.

  - The position and general requirements for this class of advertising signs are as follows:

    - General requirements as stated in Section 14 to 21 apply.

    - Standardized pole-mounted posters may be permitted only where they will not have a negative visual impact on the streetscape and the character of an area.

    - Only permitted on electric light standards or other structure, which is provided for the express purpose of affixing posters.

    - At least 50m from the centre of an intersection.

    - May not be placed on or against or attached to or otherwise supported by any power line standard, power masts, transformer box, telegraph pole, road traffic sign or signal (any lamp pole which has a road traffic sign or signal attached to it), traffic circle, traffic island or median (except Category Two (2) and Three (3) Posters), wall, column or post of a veranda or balcony, fencing, electricity box or sub-stations, tree or bridge.

    - No advertising sign may be mounted on a short (4,5m) streetlight pole.

    - Not to cover municipal markings / stripes on lampposts.

    - No steel or aluminium ladders may be placed against the standards on which the posters are to be erected.

    - These advertising signs may not be illuminated or animated.

U) Only permitted inside urban road reserves for roads under the jurisdiction of the Municipality and exclude freeways, National roads and Provincial roads.

- Not closer than 0.3m from the nearest edge of any posters to the edge of the shoulder of the road.

  - Not to obstruct pedestrian movement.

- May not in any way interfere with the sight distance of motorists.

- Advertisements may not have any letters smaller than 50mm in height.

- If so required by the Municipality, the content may be subject to the Municipality's approval.

- This class is subject to the approval of the Municipality, except category five posters (for elections).

  - Conditions applicable to Category One (Events) posters:

    - A maximum of 100 posters permitted per event per Municipal area.

    - Temporary posters must be fixed to electric light standards and fixed receptacles by means of a suitable cord and no metal clamps or wire may be used.

    - May not be used to advertise a commercial event or product or sales promotion.

    - A completed street name list on where the posters will be erected, is to be provided by the applicant, within at least three days prior to the event, occasion, function or meeting as indicated on the poster. Failure to submit this list will result in the forfeiting of the deposit paid.

    - Posters must be erected only 14 days prior to the event.

    - The display period during the event shall not exceed 30 days.

    - All posters, backing boards and cord or string must be removed within 3 days of the passing of the event. Failure to remove a poster, will result in forfeiting the deposit for that poster.

    - A maximum of two (w) per post or standard. It must then be affixed in a double-sided manner back to back; i.e. only one poster visible per direction of travel.

    - The content is subject to the approval of the Municipality. The name of the relevant responsible organization, and the date and place of the occasion or event must be clearly displayed on the poster. The Municipality shall be entitled to retain one such poster for identification purposes.

U)  No products may be advertised on a Category One poster.

- Poster advertising signs aimed at the road used may not be less than 120m apart.

  - Every poster for which permission is granted, shall display a municipal sticker with a reference number.

- No poster or other advertising sign may be placed in a street or other public place unless the appropriate tariff has been paid to the Municipality.

- Every deposit paid must be refunded when all the posters, advertising signs or advertisements to which the deposit relates, have been removed to the satisfaction of the Municipality.

- Any person who, having displayed or caused to be displayed any advertising sign or advertisement, fails to remove it or cause it to be removed within the periods prescribed shall be guilty of an offence and must, in addition to any penalty imposed upon him, forfeit the deposit relating to it or such proportionate part of that deposit as the Municipality shall assess having regard to the number of posters, advertising signs or advertisements not removed.

  - Conditions applicable to Category Two (Commercial) posters:

    - Posters must be fixed to electric light standards by means of removable brackets or strapping. No drilling or welding of poles will be permitted.

    - A maximum of two per post or standard. It must then be affixed in a double-sided manner; i.e. two receptacles back to back.

    - May only be places in certain specific areas and along certain arterial roads.

    - More creative and visually pleasant structures should be used for displaying of posters than standardized pole-mounted structures in order to make a positive contribution to streetscaping.

    - The Municipality must determine the number and display format of posters in this category.

  - Conditions applicable to Category Three (Newspapers) posters:

    - Poster frames must be fixed to electric light standards by means of removable brackets or strapping. No drilling or welding of poles will be permitted.

    - May not be used to advertise a commercial event.

    - May be displayed along specific main traffic routes.

    - If projecting over a sidewalk and pedestrian circulation route, the clear height must exceed 2,4m and for a cycle circulation route a clear height of at least 3,0m;

    - A specific news headline must only be displayed for 24 hours.

    - A maximum of one per post or standard.	Back to back at same position will be permitted.

    - The Municipality must determine the number and display format of posters in this category.

    - No poster frame or news headline may be placed in a street or other public place unless the appropriate tariff has been paid to the Municipality.

  - Conditions applicable to Category Four (Community) posters:

    - Posters must be fixed to electric light standards by means of removable brackets or strapping. No drilling or welding of poles will be permitted.

    - May not be used to advertise a commercial event.

    - A maximum of one per post or standard.

    - All posters, backing boards and cord or string must be removed within 3 days of the passing of the event

    - Poster advertising signs aimed at the road used may not be less than 120m apart.

    - No poster, advertising sign or advertisement may be placed in a street or other public place unless the appropriate tariff has been paid to the Municipality.

    - Public awareness and community-based campaigns are to be directed at the residents within a specific community aimed at indicating crime statistics and reporting of incidents.

    - The Municipality must determine the number and display format of posters in this category.

  - Conditions applicable to Category Five (Elections) posters:

    - Posters must be fixed to electric light standards and fixed receptacles by means of a suitable cord and no metal clamps or wire may be used.

    - May not be used to advertise a commercial event.

    - A maximum of three per post or standard.

    - The Municipality must in the absence of legislative prescriptions determine the number and display format of posters in this category.

    - No posters relating to a parliamentary or municipal election, referendum or registration process may be displayed for longer than the period extending from the beginning of the date of proclamation in the Government Gazette of an upcoming referendum or election to the end of the fourteenth day after the date of such election, referendum or registration process.

# Advertisements on litter bins

  - Advertisements on litter bins will be permitted in areas of maximum, partial and minimum control.

  - The size of advertisements on litter bins must be smaller or equal to 2,2m2 in total area, provided that the different advertisements face more than one direction

  - The position and spacing requirements for this class of advertising signs are as follows:

    - Permitted inside urban road reserve under the jurisdiction of the Municipality, excluding all Provincial and National roads.

    - May not be positioned closer than a minimum of 1,8m from the edge of the pavement litterbin to the edge of the shoulder of the road; and 0,3m from a cycle path, footpath or sidewalk. It may not be placed in such a way that it may cause deviation of the normal and easy flow of pedestrians or cyclists.

    - Not to obstruct pedestrian movement.

    - May not in any way interfere with the sight distances of motorists.

    - May not be placed on a road median.

    - Pole-mounted litter bins are only permitted on streetlight poles or other structure, which is provided for the express purpose of affixing pole-mounted litter bins.

    - Pole-mounted litter bins may not be attached to power line standard, power masts, road traffic sign or signal, traffic circle, traffic island, wall, column or post of a veranda or balcony, fencing, electricity box or sub-stations, tree or bridge.

    - Only one pole-mounted litterbin will be permitted per streetlight pole and one advertisement will be permitted per pole-mounted litter bin.

    - No drilling or welding of poles will be permitted. Pole-mounted litter bins must be fixed to poles by means of removable brackets or stripping approved by the ED.

  - Litter bin advertisements may not be illuminated or animated.

  - This class consists of advertising on pavement and pole-mounted litter bins, which are not intended primarily for advertising but which are provided for pedestrians and commuters.

  - General requirements as stated in Section 14 to 21 apply.

  - It provides ample opportunity for non-locality bound advertising along urban roads and streets inside road reserves as well as opportunities for non-locality bound advertising in public spaces and in other pedestrian-orientated areas at shopping centers, shopping malls and at transport nodes.

  - Litter bins may not be used or positioned for the primary or sole purpose of advertising, but only where warranted by pedestrian movement, or at locations as requested by the Municipality.

  - This class is subject to the approval of the Municipality.

# Advertisements on public transport shelters

  - Advertisements on public transport shelters will be permitted in areas of maximum, partial and minimum control.

  - The size and height of advertisements in this class will be guided by the design of the public transport shelter, and will be subject to Municipal approval.

  - The position and spacing requirements for this class of advertising signs are as follows:

    - Permitted only on public transport shelter which are inside urban road reserve or on any other property under the jurisdiction of the Municipality, excluding all Provincial and National roads.

    - May not be positioned closer than 1,8m from the nearest edge of the public transport shelter to the edge of the shoulder of the road, but can be reduced to a minimum of 1,2m if approved by the Municipality; and 0,3m from a cycle path, footpath or sidewalk.

    - Public transport shelters must be placed at specific locations as approved by the Municipality, on a public transport route where there is a need for such a shelter.

    - Not to obstruct pedestrian or cyclist movement.

    - May not in any way interfere with the sight distances of motorists.

    - May not be placed on a road median, road or traffic island.

  - Advertisements on public transport shelters may be illuminated, subject to Section 17, but not animated. No illumination may be permitted in areas of maximum control.

  - This class consists of advertisements on public transport shelters which forms part of the public transport shelter structure. The public transport shelter is a free-standing covered structure at a bus stop, taxi rank or lay-bye, with the purpose to provide a limited shelter for commuters and pedestrians, and is not intended primarily for advertising.

  - General requirements as stated in Section 14 to 21 apply.

  - The advertisement must be affixed to, and form part of, the public transport shelter, and may not be a free-standing structure. No part of the advertising sign or advertisement may extend beyond the panels or roof of the public transport shelter, as approved by the Municipality.

  - Public Transport Shelters must be constructed in accordance to the Municipality's approved specifications and designs.

  - This class is subject to the approval of the Municipality.

# Stack signs

  - Stack signs will only be permitted in areas of minimum control, and in other areas of control, only if approved by the relevant road authority.

  - The size and height of advertising signs allowed in this class, are as follows:

Size:

Height:

Maximum 16m2.

Maximum size may be increased to 23m2 if height restriction is relaxed in terms of the Town Planning Scheme or any other relevant document.

Minimum dimension per panel: 1m x 1m.

Not less than four and not more than ten panels per side. Maximum 7,5m.

Height may be increased to 10,5m if permitted in the Town Planning Scheme or any other relevant document.

Width of structure: Not less than 2,1m

Not more than 3m

Clear Height:	2,4m

  - A maximum of two stack sign per intersection, positioned diagonally, will be permitted.

  - A stack sign may be illuminated, subject to Section 17, but may not be animated.

  - This class consists of a free-standing advertising sign in the road reserve displaying a combination of advertisements of several businesses operating in the direct vicinity, on separate panels.

  - General requirements as stated in Section 14 to 21 apply.

  - Advertisement panels may not be reflective.

  - This class is subject to the approval of the Municipality.

# Street name signs

  - Street name signs will be permitted in area of maximum, partial and minimum control.

  - The size and height of advertising signs allowed in this class, are as follows: Street name sign (GL1 signs):

Rectangular in shape (according to SADC RTSM requirements).

Minimum Length:	500mm.

Maximum Length:

Width (height): Advertisement panel:

Determined by the street name length and type of sign.

300mm.

Rectangular in shape, either in a landscape or portrait format. Minimum Area:	1,00 m2.

Maximum Area:	1,68 m2.

Horizontal measurement: Between 0,8m and 1,6m. Vertical measurement:	Between 1,0m and 1,6m.

Clear Height:	Minimum of 2,1m to the street names.

Only one advertisement per side will be permitted.

  - The position and spacing requirements for this class of road sign combined with an advertisement panel are as follows (should also adhere to SADC RTSM, GL1 sign requirements):

    - Street name section should be below advertising panel, but not closer than 200mm.

    - May not extend over the road surface.

    - It may not be located directly in front of, or behind a road traffic signal.

    - Must be located as far as possible in the far left opposite corner of the junction of two streets, for both directions of traffic, specifically for the main roads. It will therefore be positioned diagonally.

    - Maximum two illuminated advertising signs diagonally opposite one another per intersection. At junctions of lower order roads, one street name sign (combined with an advertisement panel) will normally be adequate.

    - Inside urban road reserve (Municipal roads only), but not on any Provincial or National road, road medians or road islands, and it should take into consideration the traffic movements, other road traffic signs and street furniture.

    - May not be erected closer than a minimum of 0,5m from the edge of the advertising sign to the vertically projected edge of the shoulder of the road; and 0,3m from a cycle path, footpath or sidewalk.

    - May not interfere with the sight distances of motorists or obstruct pedestrian movement.

  - Internal illumination of the advertising sign may not exceed the luminance intensity of the street name section. Illumination is permitted subject to Section 17. These advertisements may not be animated and may not flash.

  - This class consists of pole-mounted street name signs (road traffic signs - GL1 sign in the SADC RTSM), carrying a double-sided advertisements, which may be internally illuminated, displayed in combination with street name signs in the urban environment.

  - General requirements as stated in Section 14 to 21 apply.

  - The street name must be in black letters on a white background.

  - Any street name on the advertisement panel must be smaller and less conspicuous than the street name on the actual street name panel.

  - The layout of the advertisement must be of such a nature that it may not be misunderstood to represent a road traffic sign, due to any factors such as:

    - a combination of colours and type of arrows as used for road traffic signs,

    - symbols used on road traffic signs, or

    - a combination of colours specified for road signs.

  - The layout of the advertisement may also not encourage U-turns.

  - Only one advertisement per side of the advertising sign, will be permitted. Each advertisement, per side, may not be split to accommodate two (2) or more advertisements.

  - This class is subject to the approval of the Municipality.

# Suburb name signs

  - Suburb name signs will be permitted in areas of maximum, partial and minimum control.

  - The size and height of advertising signs allowed in this class, are as follows:

GL2 sign:	Rectangular in shape (according to SADC RTSM requirements) Width:	1,8m

Length:	0,4m

Minimum letter height:	175mm (may be reduced to 140mm for lower order roads)

Advertisement panel:

Rectangular in shape Width:

Length:

Not wider than suburb name sign 0,4m

Maximum length:	0,5m

The advertisement must be less conspicuous than the suburb name.

Height of the advertising sign:	Maximum 4m

  - A clear height of not less than 2,1m must be allowed for between the bottom of the sign and the ground, irrespective of the size of the sign.

  - The position and spacing requirements for this class of advertising signs must be according to SADC RTSM. It must be placed on the left side of the roadway at the approximate point of entrance into a suburb along the highest class of road entering a suburb, taking into consideration the location of junctions, other road traffic signs and street furniture. It may not be located on road medians or islands. Extensions of a larger suburb do not warrant suburb name signs.

  - May not be erected closer than a minimum of 0,5m from the edge of the advertising sign to the vertically projected edge of the shoulder of the road; and 0,3m from a cycle path, footpath or sidewalk, and at least 50m from the centre of an intersection. It may not interfere with the sight distances of motorists or obstruct pedestrian movement.

  - As these advertising signs are attached to suburb name sign, which are road traffic signs, their positioning shall be dependant on the positioning of the suburb name sign.

  - Internal illumination is permitted, subject to Section 17. These advertisements may not be animated and may not flash.

  - This class consists of pole-mounted location signs (road traffic signs - GL2 sign in the SADC RTSM) at entrances to towns and suburbs, carrying an advertisement beneath the suburb name.

  - General requirements as stated in Section 14 to 21 apply.

  - The sign must be divided horizontally. The suburb name must appear in the top panel on the front side, and must be done according to prescriptions. The reverse side of the top panel must be clear of any advertisement. The top panel may be combined with an advertising panel underneath the suburb name signs. The front side of the advertising panel may contain non-locality bound advertising. The reverse side of the advertising panel may be utilised for the promotion of community service messages, non-profitable organisations, emergency services, or branding of the Municipality, as provided for and required by the Municipality, or alternatively if not used by the Municipality, for non-locality bound advertisements.

  - Suburb name signs are permitted within all urban road reserves under the jurisdiction of the Municipality, excluding all freeways, Provincial and National roads.

  - No colours that may cause confusion with road traffic signs may be used.

  - The background of the advertising sign may not be retro-reflective or fluorescent.

  - This class is subject to the approval of the Municipality.